Originals/Modifications

Keep in mind that modifications may have been made since these notes were written.

Modifications

VJob Patch

Originals

Me Am Recto (missing patch)

Just Shred

8-bit 1

8-bit 2

8-bit 3

Additional Originals

Foo Fighters Colour and the Shape

Dub Step

# Modifications

## V-Job - Ola's H-Job Patch

## OR_VJob_1113

#### Overview

Basically I'm using Ola's H-Job Patch but I'm making it dual cab to add some quality to it.

#### Amp/Distortion Tone

It's pretty much a standard Screamer > ENGL tone, similar to Periphery and Scar Symmetry.

#### Cab and Mic

Ola's patch used the XXL/409 cab/mic. I'm using that combined with the Treadplate/57 on axis.

# Originals

## MeAmRecto

## ??? (missing patch)

#### Overview

What I wanted to do hear was slightly tweak a natural Rectifier tone. I'm not trying to dial in anything too unique. I just use all the research I've put into the Pod HD to create a killer Rectifier patch.

#### Amp/Distortion Tone

A slight mid-boost to clean up the tone just a little bit. Otherwise, it's just the awesome power of modeled Mesa.

#### Cab and Mic

I use a Treadplate/57 on axis paired with a Hiway/4038. This is my favorite dual cab combo. It gets you a great frequency response with nothing sounding washed out or noisy, but it's not too harsh either.

#### Cab DEP's

I boost the Low Cut on the Treadplate cab, so that its low end doesn't interfere with the Hiway's.

## Just Shred

## OR_JstShrd_1208

#### Overview

I wanted to dial in a really tight, yet smooth, modern lead tone.

#### Amp/Distortion Tone

Just using the raw Fireball model. I find it has a great tone for leads without much tone-tweaking in front of it.

#### Cab and Mic

I use a Treadplate/57 on axis paired with a Hiway/4038. This is my favorite dual cab combo. It gets you a great frequency response with nothing sounding washed out or noisy, but it's not too harsh either.

#### Cab DEP's

I boost the Low Cut on the Treadplate cab, so that its low end doesn't interfere with the Hiway's.

#### EQ

The Fireball model is naturally a bit midsy, which is perfect for a lead tone that cuts through the mix.

## 8-bit 1

## OR_8bit1_L

#### Overview

Going for one of those synth-type tones you'd hear on a lead line on NES. This will only work for single-note leads - chords will sound like unpredictable madness.

#### Amp/Distortion Tone

I use some Distortion effects and a Marshall amp for some compression and dirt in the tone, but the real driver of tone is the synth effect listed below.

#### Amp DEPs

Just adjusting Master to get the right amount of distortion.

#### Cab and Mic

Hiway/57 off axis. Simple but good - I don't need to worry about low-end response.

#### Effects

I use a gate to make it sound more like a synth - silent when not playing. The synth effect is what drives the tone here. It gets you that super smooth synth sound. A touch of reverb to soften it up.

## 8-bit 2

## OR_8bit2_L

#### Overview

Instead of using the synth effect here, I use a pitch shifter a perfect fifth downward. This kind of creates a synth-like tone, but it's much more responsive than using the actual synth effect.

#### Amp/Distortion Tone

Again, a Marshall distortion tone.

#### Effects

Pitch Glide set to a fixed perfect fifth downward to get the synth effect. Gate. Reverb.

## 8-bit 3

## OR_8bit3_L

#### Overview

Just combining both techniques from above. Synth in Channel A. Pitch-shift in Channel B. It all mushes together surprisingly coherently.

# Additional Originals

## Foo Fighters Colour and the Shape

## OR_FooFgt_L

## Dub Step

## OR_DubStep_L